WEARABLE NAVIGATION SYSTEM

Full Study Material & Technical Viva Guide

YOLOv8n • Multi-object Tracking • Distance Calibration • GRU Risk Prediction • MiDaS • OpenVINO • ESP32 + HC-SR04

CURRENT FINAL RESULT Held-out real-world test: 90.95% overall bucketed accuracy, 88.07% macro accuracy, MSE 0.0375. This result comes from 5 completely held-out real sessions and must be reported as the final test result.

Prepared from the project source files and the latest evaluation output supplied in this session.

# How to Use This Study Material

This document is designed to let you explain the project from first principles, defend the machine-learning methodology, understand the live pipeline, and answer technical viva questions. It follows the terminology and implementation choices in the supplied project files rather than replacing them with a generic architecture.

Study Sections 1–6 for the core computer-vision and ML concepts.

Study Sections 7–10 for the real-data methodology, evaluation, and interpretation.

Study Sections 11–13 for the MiDaS unknown-obstacle layer, acceleration, and hardware integration.

Use Sections 14–16 immediately before a project demonstration or viva.

The 'Know These Numbers' boxes contain the exact implementation values worth memorizing.

IMPORTANT REPORTING RULE Do not call a validation score the final test score. The final score is the result from the locked test sessions evaluated separately after training.

Source basis: train_on_recorded_sessions(5).py; evaluate_on_recorded_sessions(5).py

# 1. Project Objective and Core Idea

The system is a wearable navigation assistant intended to identify obstacles, estimate their spatial/temporal risk, and produce a compact LOW / MEDIUM / HIGH risk decision suitable for a navigation aid.

The central design idea is not simply 'detect the nearest object'. The system tracks multiple objects, estimates closing speed and time-to-collision (TTC), and feeds the most urgent tracked obstacle into a recurrent GRU model. A separate MiDaS-based layer searches for visually salient obstacles that YOLO cannot semantically classify.

| Layer | Purpose | Main technology |
| --- | --- | --- |
| Visual sensing | Acquire the current scene | Phone/IP camera + OpenCV |
| Semantic detection | Recognize known COCO objects | YOLOv8n |
| Tracking | Maintain object identity across frames | Centroid + class matching |
| Metric distance | Convert bbox size to distance | Power-law calibration |
| Motion reasoning | Estimate closing speed and TTC | Temporal history + filtering |
| Risk prediction | Predict continuous 0–1 risk | 2-layer GRU |
| Unknown obstacle detection | Find prominent depth protrusions | MiDaS + temporal stability |
| Acceleration | Reduce inference latency | OpenVINO GPU |
| Physical ranging | Provide direct distance measurement | ESP32 + HC-SR04 + UDP |

Key conceptual distinction YOLO answers 'what is this?'; distance estimation answers 'how far?'; tracking answers 'how is it moving?'; TTC answers 'how urgent is it?'; GRU answers 'what risk pattern is emerging?'; MiDaS answers 'is there a visually meaningful obstacle even if YOLO has no class for it?'

Source basis: yolo_webcam_scaffold(8).py; yolo_webcam_scaffold_yolo_gru_midas_unknown_integrated(4).py

# 2. End-to-End System Architecture

A useful mental model is the following pipeline:

| Stage | Input | Output | Decision role |
| --- | --- | --- | --- |
| 1. Camera | MJPEG video stream | Latest frame | Fresh visual evidence |
| 2. YOLO | Current frame | Objects + bounding boxes | Known-object semantics |
| 3. Tracker | Detections over time | Tracks | Stable object histories |
| 4. Distance | BBox area | Metric distance | Spatial proximity |
| 5. Motion | Distance history | Closing speed + TTC | Threat urgency |
| 6. Selection | Top tracked obstacles | Lowest-TTC obstacle | GRU target |
| 7. GRU | 30 × 4 feature sequence | Risk score 0–1 | Predictive risk |
| 8. Safety layer | GRU + proximity + MiDaS | Final risk/bucket | Conservative safety decision |
| 9. Output | Final decision | LOW/MEDIUM/HIGH + reason | Navigation feedback |

## 2.1 Why the latest-frame reader matters

The camera stream can buffer frames faster than the main inference loop processes them. If old frames accumulate, the system gradually operates on stale imagery. The project therefore uses a background reader that continuously decodes the stream and retains only the newest valid frame. This keeps processing close to real time.

LatestFrameReader

camera thread -> continuously read frames

-> overwrite latest_frame

main loop    -> read() -> newest available frame

Design principle For a navigation system, low latency is not just a performance optimization. A prediction based on an old frame can be a safety-relevant error because the obstacle may have moved substantially since capture.

Source basis: calibrate_distance(5).py; yolo_webcam_scaffold(8).py; integrated YOLO+GRU+MiDaS file

# 3. YOLOv8n: Semantic Object Detection

YOLOv8n is used as the semantic detector. The detector returns bounding boxes and class identities for objects within the model's supported COCO vocabulary. The current project intentionally preserves native YOLO class names inside the integrated navigation pipeline, while a legacy six-class mapping is applied only at the GRU boundary.

| Concept | Meaning in this project |
| --- | --- |
| Bounding box | Rectangle around a detected object: x1, y1, x2, y2 |
| Class | Semantic identity such as person, car, chair, bottle, etc. |
| Confidence threshold | 0.25 in the integrated OpenVINO configuration |
| NMS IoU | 0.45 in the integrated configuration |
| Inference size | 256 for the live OpenVINO path |
| Range | Distance estimates are clipped to a maximum of 4.0 m |

## 3.1 Important limitation: detector vocabulary

A mapping dictionary cannot create new detector capabilities. If an object is not one of YOLOv8n's learned COCO classes, adding its name to CLASS_MAP does not make the pretrained model detect it. A custom-trained detector would be required to add genuinely new semantic classes.

## 3.2 Why multi-object tracking?

Selecting only the physically nearest object is not sufficient. A farther vehicle approaching quickly can have a lower TTC than a closer stationary pole. The current tracker maintains up to three simultaneous tracks and selects the tracked obstacle with the lowest TTC for GRU input.

Source basis: yolo_webcam_scaffold(8).py; yolo_webcam_scaffold_yolo_gru_midas_unknown_integrated(4).py

# 4. Distance Estimation and Calibration

The original camera-only distance estimate is derived from the fraction of image area occupied by the detected bounding box. Because bbox area is not a perfect geometric measurement, the project fits an empirical power law.

Core equation distance = C × (area_frac)^(-p)

The calibration tool asks the user to stand at known distances, captures the freshest camera frame, measures the largest person's bbox area fraction, and fits C and p using log-log linear regression.

Stand at a known distance, for example 0.50 m.

Wait for the displayed area fraction to stabilize.

Press SPACE and enter the true distance.

Repeat at multiple distances, e.g. 0.50, 1.00, 2.00 and 3.00 m.

Fit the power law using the collected (distance, area_frac) points.

Inspect per-point errors; add more calibration points if errors remain large.

## 4.1 Why not assume exponent 0.5?

A naive area-vs-distance geometric argument suggests a square-root relationship. The project deliberately does not force that exponent because lens distortion, bounding-box tightness, object pose, and close-range clipping can shift the empirical relationship.

| Current integrated value | Value | Meaning |
| --- | --- | --- |
| DIST_C | 0.4327 | Power-law scale coefficient |
| DIST_EXPONENT | 0.7530 | Empirically fitted exponent |
| Calibration span | 0.25–3.00 m | 12 fresh person-distance points |
| Distance clipping | 0.20–4.00 m | Operational range of the estimator |

Calibration dependency If the camera, mounting geometry, or relevant image-processing setup changes, distance calibration should be rechecked. The integrated code explicitly treats these coefficients as calibration-dependent.

Source basis: calibrate_distance(5).py; yolo_webcam_scaffold_yolo_gru_midas_unknown_integrated(4).py

# 5. Multi-Object Tracking, Closing Speed and TTC

Each detection is associated with an existing track using centroid distance and class consistency. The track stores a short history of time-distance measurements. Distance is smoothed before its derivative is used.

| Quantity | Definition / role |
| --- | --- |
| Distance history | Recent metric distances for one tracked obstacle |
| Closing speed | (d0 − d1) / Δt; positive means the obstacle is getting closer |
| Deadband | Speeds with magnitude below 0.05 are treated as zero |
| TTC | distance / closing_speed when closing_speed > 0 |
| Non-closing TTC | 999.0, effectively 'not an immediate threat' |
| Speed history | Up to 8 frames in the current integrated configuration |
| Track timeout | 1.0 s without a match |
| Maximum tracks | 3 simultaneous tracks |

TTC selection rule The selected obstacle is the tracked obstacle with the LOWEST TTC. Lower TTC means greater urgency.

## Example

| Obstacle | Distance | Closing speed | TTC | Priority |
| --- | --- | --- | --- | --- |
| Stationary pole | 1.5 m | 0.1 m/s | 15 s | Lower |
| Approaching vehicle | 2.5 m | 1.5 m/s | 1.67 s | Higher |

The vehicle is farther away but is more urgent because its time-to-collision is much smaller. This is the key reason TTC is a stronger selection criterion than raw nearest-distance.

Source basis: yolo_webcam_scaffold(8).py; integrated YOLO+GRU+MiDaS file

# 6. GRU Risk Prediction

The GRU is a sequence model. Instead of treating each frame independently, it receives a 30-timestep sequence and learns temporal patterns in distance, motion, object category and assumed agent speed.

| Input feature | Representation |
| --- | --- |
| Distance | Distance normalized by MAX_RANGE (4.0 m) |
| Closing speed | Current estimated approach rate |
| Class encoding | Legacy six-class categorical encoding scaled by /5 |
| Agent speed | Assumed agent speed, currently 1.2 |

## 6.1 Model architecture

Input:  (30 timesteps, 4 features)

↓

GRU(32, return_sequences=True)

↓

GRU(16, return_sequences=True)

↓

Dense(1, sigmoid)

↓

Risk score in [0, 1]

The sigmoid output naturally constrains the predicted risk to approximately the 0–1 interval. Training uses mean squared error (MSE), because the model predicts a continuous risk value rather than a direct three-class label.

## 6.2 Risk buckets

| Risk score | Bucket |
| --- | --- |
| risk < 0.33 | LOW |
| 0.33 ≤ risk < 0.66 | MEDIUM |
| risk ≥ 0.66 | HIGH |

Why report both MSE and bucketed accuracy? MSE measures numerical closeness of the continuous prediction. Bucketed accuracy measures whether the final LOW/MEDIUM/HIGH decision matches the ground truth. They answer different questions and should not be treated as interchangeable.

Source basis: train_risk_gru(4).py; train_on_recorded_sessions(5).py; evaluate_on_recorded_sessions(5).py

# 7. Synthetic Pretraining and Rebalanced Data

The project first uses synthetic sequences to develop a risk model before fine-tuning it on real camera sessions. The synthetic generator was specifically rebalanced because the original random generator produced an overwhelming majority of high-risk timesteps.

| Scenario | Typical synthetic setup | Purpose |
| --- | --- | --- |
| SAFE | 2.5–4.0 m start; obstacle mostly moving away/neutral | Learn genuinely low-risk behavior |
| AMBIGUOUS | 1.2–2.5 m; moderate closing speed | Learn borderline decisions |
| DANGEROUS | 0.5–1.5 m; faster closing | Learn genuine collision-course patterns |

The generator explicitly chooses scenario type first in approximately equal thirds, then samples parameters that are likely to produce that scenario. This is stratified scenario generation: the desired class balance is controlled directly instead of emerging accidentally from random physics.

Synthetic X shape: (4000, 30, 4)

Synthetic y shape: (4000, 30)

X[t] = [distance / 4.0, closing_speed, class / 5.0, agent_speed]

risk = clip(1.5 / (TTC + 0.3), 0, 1)

Methodological caution Synthetic performance is a development result, not proof of real-world performance. The project therefore adds a real-session collection, session-level split, fine-tuning stage, and locked held-out test.

Source basis: synthetic_dataset_generator(3).py; train_risk_gru(4).py

# 8. Real-World Session Collection and Ground Truth

Real validation sessions are recorded with the same feature computation used by the live pipeline. This is important: the model is not being evaluated on a simplified offline feature extractor that differs from deployment.

| Session type | Recording behavior |
| --- | --- |
| SAFE | Move around/near objects without an actual collision approach; end without a collision marker. |
| DANGEROUS | Move toward an obstacle until the instant an actual collision/near-miss would occur; press 'c'. |
| AMBIGUOUS | Approach an object but veer away before contact; no collision marker required. |

## 8.1 Event-based ground truth

When a future collision marker exists, the recorder computes the real time-to-collision from the marker timestamp rather than relying only on estimated distance and speed. The risk is then derived as:

Ground-truth event formula risk = clip(1.5 / (real_ttc + 0.3), 0, 1)

For frames with no upcoming collision marker, the recorder falls back to the same instantaneous distance/closing-speed risk formula used by the synthetic generator, but using real sensor-derived values.

## 8.2 Session file structure

Sessions are saved as .npz files containing the feature sequence, risk labels, and session metadata. The session label is organizational; the actual target is generated by the recording logic.

Source basis: record_validation_sessions(4).py

# 9. Training / Validation / Final Test Methodology

The critical methodological improvement is splitting by whole session before constructing overlapping sliding windows. If windows were split randomly first, adjacent overlapping windows from one recording could appear in both training and test data, causing leakage and overstating generalization.

| Set | Used for | Allowed during training? |
| --- | --- | --- |
| TRAIN | Update GRU weights | YES |
| VALIDATION | Checkpoint/model selection | YES, for evaluation only |
| FINAL TEST | One-time held-out real-world evaluation | NO |

## 9.1 Current run

| Split | Sessions |
| --- | --- |
| Train | 25 |
| Validation | 5 |
| Final test | 5 |
| Final test windows | 412 |
| Window timesteps | 12,360 |

## 9.2 Fine-tuning strategy

The real-session training script loads the synthetic-pretrained model and fine-tunes it using a lower learning rate (5e-4). Training uses timestep-level sample weights derived only from training labels. Validation is checked periodically.

Checkpoint selection prioritizes the safety-critical medium and high buckets: the validation safety score is 50% medium accuracy + 50% high accuracy. Validation macro accuracy and validation MSE are used as tie-break signals.

Why this is defensible The final test set is not used for gradient updates, checkpoint selection, early stopping, or hyperparameter selection. The evaluation script independently loads the locked test list from session_split.json and checks that every evaluation window belongs to that set.

Source basis: train_on_recorded_sessions(5).py; evaluate_on_recorded_sessions(5).py

# 10. FINAL HELD-OUT REAL TEST RESULTS

These are the numbers to report for the current experiment. They are the results from the five locked real-world test sessions supplied in the evaluation output.

| Metric | Final result |
| --- | --- |
| MSE | 0.0375 |
| Overall bucketed accuracy | 90.95% |
| LOW accuracy | 91.64% (n = 10,801) |
| MEDIUM accuracy | 81.84% (n = 804) |
| HIGH accuracy | 90.73% (n = 755) |
| Macro accuracy | 88.07% |
| Prediction spread | min 0.009; max 0.977; std 0.2095 |

## 10.1 Confusion matrix

| True \ Predicted | LOW | MEDIUM | HIGH |
| --- | --- | --- | --- |
| LOW | 9,898 | 650 | 253 |
| MEDIUM | 32 | 658 | 114 |
| HIGH | 0 | 70 | 685 |

## 10.2 How to interpret the result

90.95% is the correct overall bucketed accuracy for the final held-out real test.

88.07% macro accuracy is lower because macro accuracy gives LOW, MEDIUM and HIGH equal importance rather than letting the abundant LOW class dominate.

MEDIUM is the weakest bucket at 81.84%, indicating that borderline cases remain the hardest classification problem.

HIGH accuracy is 90.73%, and the matrix shows 0 true-HIGH samples predicted as LOW. This is an important safety-relevant property of this test run.

The model still confuses some LOW cases with MEDIUM/HIGH, which can produce conservative warnings. It also confuses some MEDIUM cases with HIGH, showing that the boundary region is difficult.

Target honesty If the project guide has a target such as >92% overall accuracy, the current 90.95% result should not be described as meeting that target. It is better to report the measured result and explain the remaining weakness, especially the MEDIUM bucket.

## 10.3 Exact test sessions

| Session | Frames | LOW | MEDIUM | HIGH |
| --- | --- | --- | --- | --- |
| ambiguous3 | 125 | 100.0% | 0.0% | 0.0% |
| ambiguous6 | 341 | 80.4% | 13.8% | 5.9% |
| dangerous21 | 113 | 23.9% | 26.5% | 49.6% |
| safe1 | 545 | 100.0% | 0.0% | 0.0% |
| safe4 | 256 | 92.6% | 7.4% | 0.0% |

Source basis: User-supplied final evaluation output; evaluate_on_recorded_sessions(5).py

# 11. MiDaS: Unknown-Obstacle Detection

YOLO's strength is semantic recognition, but it has a fixed learned vocabulary. The integrated system therefore adds MiDaS monocular depth estimation as a complementary mechanism for visually prominent obstacles that do not have a recognized YOLO class.

| Component | Current implementation |
| --- | --- |
| Model | MiDaS v2.1 Small |
| Runtime | OpenVINO |
| Device | GPU |
| Input | 256 × 256 |
| ROI | Lower/forward image region, approximately 34%–92% of image height |
| Navigation zones | LEFT / CENTER / RIGHT |
| Temporal confirmation | 3 of last 5 frames |
| Box smoothing | EMA alpha 0.65 in standalone detector |
| Bottom-edge handling | Ignore extreme bottom area because floor/body can dominate depth |

## 11.1 Relative depth versus metric distance

MiDaS provides relative depth rather than a direct metric distance in metres. The integrated implementation therefore does not blindly reuse the YOLO bbox-distance calibration for MiDaS boxes. Instead, it learns a separate metric mapping from matched YOLO+MiDaS observations.

## 11.2 Unknown-obstacle safety floor

Confirmed MiDaS-only protrusions are integrated after the GRU rather than changing the trained GRU's four-feature schema. This preserves compatibility with the existing model while allowing an unknown visual obstacle to raise the final safety level.

Important architecture point MiDaS is a complementary safety channel, not a replacement for YOLO. YOLO provides semantics; MiDaS provides depth structure and a route to detecting salient obstacles outside the YOLO vocabulary.

Source basis: test_midas_obstacle_detector_standalone(4).py; test_midas_obstacle_detector_stable_low_latency_v2(4).py; integrated YOLO+GRU+MiDaS file

# 12. Deterministic Safety Layer and Live Risk Logic

The integrated live system contains a deterministic proximity override outside the GRU. This is intentional: a very close obstacle should not depend entirely on a learned temporal predictor, particularly when closing speed is near zero.

| Rule | Current value / behavior |
| --- | --- |
| Critical proximity | ≤ 0.35 m → CRITICAL |
| High proximity | ≤ 0.45 m → HIGH |
| Bottom-zone start | bbox bottom ≥ 78% of frame → at least MEDIUM |
| Bottom-zone critical | bbox bottom ≥ 95% of frame → HIGH |
| GRU HIGH boundary | risk ≥ 0.66 |
| GRU MEDIUM boundary | 0.33 ≤ risk < 0.66 |

## 12.1 Risk combination

If proximity = CRITICAL:

final risk = 1.0

If proximity = HIGH:

final risk = max(GRU risk, 0.66)

If proximity = MEDIUM:

final risk = max(GRU risk, 0.35)

Otherwise:

final risk = GRU risk

bucket(final risk) -> LOW / MEDIUM / HIGH

This is especially relevant to the issue observed in the live view where objects near the bottom of the camera could receive a medium-risk state even when the GRU itself did not strongly predict danger. The deterministic layer is the place where immediate foreground proximity is deliberately enforced.

Source basis: yolo_webcam_scaffold(8).py; integrated YOLO+GRU+MiDaS file

# 13. OpenVINO GPU Acceleration and Latency

The project exports/uses OpenVINO models for GPU inference. The supplied YOLO benchmark compiles the YOLOv8n OpenVINO model on the GPU, warms it up for 20 iterations, and measures inference latency over 200 successful inferences.

| Benchmark component | Implementation |
| --- | --- |
| Model | YOLOv8n OpenVINO XML |
| Device | GPU |
| Input size | 256 × 256 |
| Warm-up | 20 inferences |
| Benchmark | 200 successful inferences |
| Reported statistics | Average, median, best, worst latency + loop FPS |

The project also uses asynchronous MiDaS processing in the integrated system so the main loop does not have to wait for every depth result. This is another latency-management technique.

Windows TensorFlow note from the current run The supplied evaluation output reports that native Windows TensorFlow GPU support is unavailable for TensorFlow ≥2.11 in that environment, so TensorFlow inference runs on CPU there. OpenVINO GPU acceleration is therefore particularly useful for the YOLO/MiDaS vision components.

Source basis: test_yolo_openvino(3).py; integrated YOLO+GRU+MiDaS file; user-supplied evaluation console output

# 14. Hardware Integration: ESP32 + HC-SR04

The next hardware stage introduces a direct ultrasonic distance measurement path. The supplied UDP listener is already the laptop-side communication component: it waits for UDP packets from the ESP32 and parses the distance.

| Hardware/software block | Role |
| --- | --- |
| ESP32 | Reads HC-SR04 and transmits measurements |
| HC-SR04 | Ultrasonic ranging sensor |
| Wi-Fi | Carries UDP measurement packets |
| Laptop UDP listener | Receives and parses distance readings |
| Navigation pipeline | Consumes the measurement as an additional distance signal |

## 14.1 UDP packet format

distance_cm:23.4,ts:154823

The listener binds to 0.0.0.0 on UDP port 4210 and accepts the distance_cm field. The parser rejects malformed packets rather than allowing a corrupted packet to become a valid measurement.

## 14.2 Test before real hardware

Run the listener in one terminal.

Run the built-in test sender with the --test option in another terminal.

Verify that six fake distance packets are received and parsed.

Only then wire the HC-SR04 to the ESP32 and confirm real packets arrive.

After communication is stable, fuse the ultrasonic distance with the vision-derived obstacle representation.

Hardware objective The HC-SR04 is not merely another sensor to display. Its value is that it can provide a direct metric range measurement that can stabilize or override camera-based distance estimates in the physical system.

Source basis: udp_listener(4).py

# 15. Putting Everything Together

The current conceptual deployment loop can be memorized as:

| Step | Question answered |
| --- | --- |
| Camera | What is in front of me right now? |
| YOLO | Which known objects are present? |
| Tracking | Which detections belong to the same physical objects over time? |
| Distance | How far is each tracked object? |
| Closing speed | Is it getting closer, and how quickly? |
| TTC | Which object becomes dangerous first? |
| GRU | What risk pattern is developing over the last 30 timesteps? |
| MiDaS | Is there a prominent obstacle structure without a YOLO class? |
| Proximity override | Is something already dangerously close regardless of learned prediction? |
| Final output | What LOW/MEDIUM/HIGH state should navigation communicate? |
| Hardware sensor | Can a direct ultrasonic range reading strengthen distance confidence? |

## 15.1 What is already strong

The system moved from single-object selection to multi-object tracking and TTC-based prioritization.

Camera lag was explicitly addressed using latest-frame readers.

Distance calibration was upgraded from a naive square-root assumption to an empirically fitted power law.

Synthetic training was rebalanced around SAFE / AMBIGUOUS / DANGEROUS scenarios.

Real-world validation uses the same feature pipeline as the live system.

The train/validation/test split is session-level, reducing leakage from overlapping windows.

The final test set is locked and evaluated separately.

MiDaS adds an unknown-obstacle safety channel without changing the trained GRU schema.

A deterministic proximity layer protects against immediate foreground cases.

## 15.2 What remains important to validate

Real hardware ranging and its synchronization with the camera stream.

Sensor fusion behavior when YOLO distance and HC-SR04 distance disagree.

Live GRU stability and warm-up behavior in the actual integrated application.

Medium-risk classification, which is currently the weakest test bucket.

Bottom-of-frame false positives/over-warning behavior in realistic navigation scenes.

Latency of the complete end-to-end loop, not only individual model inference.

# 16. Technical Viva Questions & Answers

Q: Why use YOLO?

A: YOLO provides fast single-stage semantic object detection with bounding boxes and learned object classes, making it suitable for real-time scene understanding.

Q: Why not simply choose the nearest object?

A: Nearest distance ignores motion. A farther object approaching quickly can have a lower TTC and therefore greater immediate risk.

Q: What is TTC?

A: Time-to-collision. In this project it is estimated as distance divided by positive closing speed. Lower TTC means higher urgency.

Q: Why use a GRU?

A: Risk depends on temporal evolution. A GRU can learn patterns across a sequence of 30 timesteps rather than treating each frame independently.

Q: Why return sequences from both GRU layers?

A: The model predicts a risk value for each timestep, so temporal outputs are preserved through the recurrent stack.

Q: Why use sigmoid?

A: It constrains the regression output to the 0–1 risk range used by the project.

Q: Why use MSE?

A: The target is a continuous risk score, so MSE measures the numerical difference between predicted and true risk.

Q: Why also report accuracy?

A: The project ultimately interprets risk as LOW/MEDIUM/HIGH, so bucketed accuracy shows whether the categorical decision is correct.

Q: Why is macro accuracy lower than overall accuracy?

A: Macro accuracy gives each bucket equal weight. Overall accuracy is dominated by the very large LOW bucket in this test set.

Q: Why split by session?

A: Sliding windows from one recording overlap heavily. Splitting by session prevents the same temporal event from leaking into both train and test.

Q: Why is the final test set locked?

A: To prevent test-set reuse for training or model selection and preserve an unbiased estimate of real-world generalization.

Q: What is the final test accuracy?

A: 90.95% overall bucketed accuracy on five completely held-out real sessions.

Q: What is the weakest class?

A: MEDIUM, at 81.84% on the final held-out test.

Q: What does the confusion matrix show about HIGH?

A: There were 685 correct HIGH predictions, 70 HIGH→MEDIUM errors, and 0 HIGH→LOW errors.

Q: Why use MiDaS if YOLO already detects objects?

A: YOLO has a fixed semantic vocabulary. MiDaS provides a complementary depth-structure signal for prominent obstacles that YOLO does not classify.

Q: Does MiDaS give metres directly?

A: No. It provides relative depth. The integrated system uses a separate metric calibration for MiDaS-only objects.

Q: Why not feed MiDaS directly into the GRU?

A: The existing GRU was trained for four input features. Keeping MiDaS outside the GRU preserves model compatibility while still allowing a safety override.

Q: Why a latest-frame reader?

A: To prevent camera buffering from causing inference on stale frames and increasing end-to-end navigation latency.

Q: Why OpenVINO?

A: It provides an optimized inference runtime and GPU execution path for the YOLO/MiDaS vision components.

Q: Why add HC-SR04?

A: It provides direct ultrasonic metric ranging, which can complement camera-based distance estimates.

Q: What is UDP doing here?

A: It transports distance measurements from the ESP32 to the laptop over the local network with a lightweight datagram protocol.

Q: What happens if YOLO cannot detect an object?

A: The MiDaS unknown-obstacle layer can still identify depth protrusions; a future custom-trained YOLO model could also expand semantic coverage.

# 17. Memorize These Numbers

| Parameter | Current value |
| --- | --- |
| GRU sequence length | 30 |
| GRU input features | 4 |
| GRU layers | 32 → 16 units |
| Risk output | sigmoid, 0–1 |
| LOW/MEDIUM boundary | 0.33 |
| MEDIUM/HIGH boundary | 0.66 |
| MAX_RANGE | 4.0 m |
| YOLO input size | 256 |
| YOLO confidence | 0.25 |
| YOLO NMS IoU | 0.45 |
| MAX_TRACKS | 3 |
| TRACK_TIMEOUT | 1.0 s |
| SPEED_WINDOW | 8 frames |
| SPEED_DEADBAND | 0.05 |
| TTC safe value | 999.0 |
| DIST_C (integrated) | 0.4327 |
| DIST_EXPONENT (integrated) | 0.7530 |
| Proximity critical | 0.35 m |
| Proximity high | 0.45 m |
| Bottom warning | 78% frame height |
| Bottom critical | 95% frame height |
| MiDaS input | 256 × 256 |
| MiDaS stability | 3 of 5 frames |
| Final test sessions | 5 |
| Final test windows | 412 |
| Final overall accuracy | 90.95% |
| Final macro accuracy | 88.07% |
| Final MSE | 0.0375 |

# 18. One-Minute Project Explanation

Recommended viva answer My system is a wearable navigation pipeline that combines semantic vision, temporal reasoning, depth estimation and physical ranging. YOLOv8n detects known objects, while multi-object tracking maintains their identities across frames. For every track I estimate metric distance, closing speed and time-to-collision, then select the lowest-TTC obstacle. A two-layer GRU processes a 30-frame sequence of four features—distance, closing speed, class encoding and assumed agent speed—to predict a continuous 0–1 risk score. I convert that score into LOW, MEDIUM and HIGH using 0.33 and 0.66 boundaries. To handle objects outside YOLO's vocabulary, MiDaS provides a depth-based unknown-obstacle channel, and a deterministic proximity layer prevents extremely close obstacles from being underestimated. The vision path is accelerated with OpenVINO GPU inference, and the next hardware stage adds an ESP32 with HC-SR04 ultrasonic ranging transmitted to the laptop over UDP. On five completely held-out real sessions, the current final model achieved 90.95% overall bucketed accuracy, 88.07% macro accuracy and MSE 0.0375.

# 19. Final Takeaways

The strongest technical story is the progression from perception → tracking → temporal risk → unknown-obstacle safety → hardware sensing.

The most important ML methodology story is session-level splitting and a completely untouched final test set.

The most important current result is 90.95% held-out real-world bucketed accuracy—not a validation score.

The most important weakness to acknowledge is MEDIUM-risk classification at 81.84%.

The next engineering milestone is hardware integration and sensor fusion, followed by end-to-end live validation.

Do not say “The model is 92%+ accurate” or “the validation score is the final result.”

Say instead “On five completely held-out real test sessions, the final model achieved 90.95% overall bucketed accuracy, with 91.64% LOW, 81.84% MEDIUM and 90.73% HIGH bucket accuracy.”

# Appendix A — Source Files Used

| Source file | Role in study material |
| --- | --- |
| synthetic_dataset_generator(3).py | Synthetic scenario generation and risk labeling |
| train_risk_gru(4).py | Initial GRU architecture and synthetic evaluation |
| record_validation_sessions(4).py | Real-world session recording and event-based ground truth |
| train_on_recorded_sessions(5).py | Session-level split, real fine-tuning and validation checkpoint selection |
| evaluate_on_recorded_sessions(5).py | Locked final held-out test evaluation |
| calibrate_distance(5).py | Fresh-frame distance calibration and power-law fitting |
| yolo_webcam_scaffold(8).py | YOLO multi-object tracking and live GRU scaffold |
| yolo_webcam_scaffold_yolo_gru_midas_unknown_integrated(4).py | Integrated YOLO + tracking + GRU + MiDaS + safety logic |
| test_midas_obstacle_detector_standalone(4).py | Standalone MiDaS depth obstacle detection |
| test_midas_obstacle_detector_stable_low_latency_v2(4).py | MiDaS low-latency/stability processing |
| test_yolo_openvino(3).py | YOLO OpenVINO GPU benchmark |
| udp_listener(4).py | ESP32/HC-SR04 UDP receiving and parsing |

# Appendix B — Methodology Notes to Keep Consistent

The final evaluation script states that the test sessions were never used for gradient updates, checkpoint selection, early stopping or hyperparameter selection.

The current console run reports 25 train sessions, 5 validation sessions and 5 final test sessions. Some older comments in the training source describe a 24/5/6 arrangement; for reporting, use the actual current manifest/run output.

If additional sessions are recorded later, create a new experiment/split rather than editing the locked test manifest.

The integrated file contains camera-only and MiDaS logic; the HC-SR04 UDP listener is currently a separate hardware communication component to be connected into the full pipeline.

Source basis: evaluate_on_recorded_sessions(5).py; train_on_recorded_sessions(5).py; udp_listener(4).py